# SC/MP Elektor Shorthand manual

For now this is a very brief documentation on how to use the SC/MP emulator. More background info on the SC/MP itself can be found at my website (http://xgistor.ath.cx) or in the Dutch and English doc-files (MS-Word).

Installation:

Unzip the ZIP-file to any drive but make sure the SCMP-folder is directly under the root. I.e. C:\SCMP

Make A shortcut to your desktop or startmenu.

Make sure that the Visual Basic 6.0 runtimes have been installed.

The Emulator Buttons:

RST = Reset CPU

HLT = When the LED burns the CPU is in halt mode and may await a click

on this button to continue. The dataswitches may be altered in between subsequent HALTs.

Run/

Prgm= Run or Program mode

When the LED is green the Emulator can by programmed by dumping

Binary data in RAM. You can use the address- & dataswitches to do so.

Every data-byte entered in Program-mode must be latched into RAM

Clicking this button will toggle between Program & Run Mode. If Single Step Mode is not active execution will commence at once. In Run mode the LED burns red.

SST= Singlestep mode is on when the LED is burning. Clicking will toggle this

mode on and off.

Latch= Will latch the data at the data-switches into the address that is determined

by the current position of the address-switches. A latch will be acknowledged by displaying that value on the red Display LEDs.

Data &

Address

Switches= Clicking on any of the small buttons will toggle the state of that bit.

Data set by the switches may be latched into the desired location by clicking LATCH. However changing a location without latching will display the RAM-value at the new location. The + & - button can be used to increment or decrement the addess-switch setting.

The value of the dataswitches may be fetched under program-control at address &H0A00 (0A00-0AFF effectively) In this case no LATCH is needed.

Green Display LEDs = Show the state of the SC/MP Flags (F2, F1, F0 respectively)

Red Display LEDs = Can be accessed by writing to location &H0A00

(0A00-0AFF effectively)

## The Menubar

Under the File Menu you can Load or Save the entire RAM-content of the emulator in a RAM-file. You can also Load a hand-written and hand assembled text-file under Load ASM-file. Some examples are aviable in the Samples subdirectory

In the settings screen you can alter some of the default setting of the emulator

Only defeat the execution-pause to 0 or a very large value if you are sure that your program runs correctly (no endless loops!) The other settings speak for themself

The trace.log file will be put in the SCMP-directory.

## Memory MAP

0000-09FF   RAM

0A00-0AFF  LEDs / DATA-switches

0B00-0FFF  Currently coded as Illegal

Only SC/MP page 0 (0000-0FFF) is implemented in the emulator.

Have Fun

Sipke de Wal

Noordwijk

The Netherlands