TI 59-58 hardware

# Annotations

### Introduction

This is the annotationfile that accompanies the TI59/58 hardware diagrams aviable on my website http://xgistor.ath.cx

This information is based upon material written in dutch by M.B. van der Mark in 1982. This material was made as a schoolproject at the time. I have never met Mr. van der Mark but I suspect that I am following his wishes if I preserve his legacy for future generations, by making it aviable on the world wide web. His thesis did not contain any legal or other restrictions for me not doing so.

Initially I will not translate all the material he has provided in this thesis, just the bare bones of the TI59 operation and a description of the signals, timings and some other internals, revamped by colorfull and comprehendable diagrams and what ever I can phantom myself but what was not directly obvious from the source material.

However, my rule of thumb for now is that I will write only about things I understand, this because some of the original information was put down in a somewhat ambiguous fashion with some loose ends that I cannot resolve at this moment. M.B. van der mark occasionally refered back to previously written material that I have no access to right now.

The annotation and the diagrams for the PC100 printer will be aviable (soon) in a separate set of files.

Anybody who is willing and able to provide additional material is invited to provide me with such and I will futher process all material and make revisions on this information, to be put in the public domain as soon as reasonably posible.

Eventually I would also like to enhance this material with stuff about the “HIR-commands” and things like “Fastmode” and “Hex-programming”. If you have any of this please let me know.

My ultimate goal would be to gather and concentrate enough information to enable a group of people to write a comprehensive emulator for the TI59/58. This would preserve this beautiful vintage calculator -in a virtual manner- for our offspring in the eons ahead.

Sipke de Wal

Springty 41

2201 WG Noordwijk

Netherlands

Email:sipke@wxs.nl

TI59/58 Block Diagram

RAM Memory

4x TMC0598

(2x TI58)

Firmware ROM

TMC 0571

Library ROM(s)

TMC 054x

Cardreader

(TI59 only)

4 r/w heads

LM324

TMC0594

Keyboard & Display

45 button keyboard &

12 character display

driven by 2x SN27882

Power supply

Adapter + internal module

Calculator CPU

Clock oscillator TMC 0301. The TMC 0501 and a piggybacked TMC 0582/0583 chipset

Printer PC100

(optional)

(2)

(1)

(3)	 (4)	(5)

(6)

Busses:

- I/O-1,I/O-2,I/O-4,I/O-8 		 4 bits Databus (Tristate)

- IDLE, 1 & 2, EXT, IRG	 Sync, Timing&Control and serial data&instructionbus

- SA … SG + DPT		 S-BUS segment lines

- D0 … D15, (L0 ..L12)		 D-BUS scanning lines (L= digit driverlines) (output)

- KN … KT			 K-BUS scanning returnlines (input)

- Vss, -Vbat, Vdd, Vgg		 Powersupply lines 0V, -3,7V, -10V, -15,7V resp.

(input/output designation with respect to the CPU)

I suspect that all the TMC-chips are PMOS-technology, this based on the fact that they all require special extra negative powersupply with respect to the main battery power. There is a thick-film powermodule in the calculator that provides for these extra voltages.

Warning! Since this powermodule does not contain any kind of voltage regulation I strongly advise you not operate the calculator without the battery package. The output of this powermodule may run out of control and may generate voltages that the PMOS-chips may not survive! In effect the batteries themselves act like a voltage regulator. (This is a feature of many calculators from the PMOS era, so be warned!)

Before we’ll cover the blockdiagram we’ll need to know more about the role of the individual chips that form the processingcore of the calculator. In the above diagram shown as the shaded parts. The rest of the machine can be regarded as periferal devices with respect to the CPU-core. To understand it’s operation we need to examine what functions are actually implemented in the core and how they are interwoven by the various types of signals.

CPU Corechips

2.5 K ROM

8 Registers

32 Constants

Program Counter

& Timing &

Control

TMC 0582

2.5 K ROM

8 Registers

32 Constants

Program Counter

& Timing &

Control

TMC 0583

TMC 0571

1K ROM

(first ROM)

TMC 0301

Dual speed

Two Phase  Clock

1 & 2

TMC 0501

Arithmetic Unit

Also generates the IDLE signal & segment lines

And accepts calculator commands from the

K-BUS

4x TMC 0598 (TI58 2x)

RAM

For Program and

Data storage

(Allocation by OP-17 command)

D-BUS

1 & 2

IDLE

IRG & EXT

4 BIT DATABUS

K-BUS

S-BUS

Based on the the above diagram we may conclude there are two sets of signal-busses in the TI59.

- The Data&Control-bus that glue the chipset together and additionally interfaces with the library-ROM, the Cardreader and the Printer. Most of the “internal” data and instructions are transmitted serially only the numerical data and the user-programmingcode are exchanged between the CPU and the RAM-chips by means of a 4-BIT unidirectional databus.

- The D-, S-, K-Scanning busses interface with the keyboard and the LED-Display. By way of exeption one of the K-lines (KR) is used as a BUSY line that is utilised by the Cardreader and the Printer to indicate their busy-state to the CPU.

The previous page shows a small version of the TI59/58 diagram (A larger version is aviable at my website)

Busses:

| Data & Control | 1 & 2 Clocksignals | This is a 2-phase (non-overlapping) clock that switches between Vss and Vgg. It is derived from a 455KHZ MF filter through the TMC0301-chip but some models of the TI59 have deviating ratings like 462KHZ. The Clock operates in two modes. - Calculating (fullspeed) - Display or IDLE-mode (lowspeed) A Clock pulse is active at the negative Vgg level and each pulse takes 1.1 uSec. Then in IDLE-mode there is a pause of 3 cycles of the clock (See timing diagram) The period between two succeeding leading egdges of 1 are defined as a “State Time”. 16 State-times make up and instruction-cycle. The first State-time of such a cycle is designated S0. Then S1, S2, S3 ….. |  | Vss-Vgg (out) |
| --- | --- | --- | --- | --- |
|  | IDLE | IDLE indicates when the Arithmetic CHIP is finished with operations and a result is displayed. IDLE switches between Vss and Vdd. IDLE will always assert negative at the start S0. If trailing edge of IDLE goes up during S1 then the calculator is in Calculating mode and works at full speed. If the trailing edge of IDLE goes up at S15 this will then indicate that the Calculator is in Display or IDLE-mode This alteration of speeds facilitates longer pulses on the D-Scanning lines at display-time in order to make results more visible and also minimises the poweruse of the CPU while the LEDs draw the most of the current. This evening out poweruse at all modes of operations. |  | Vss- Vdd (out) |
|  | IRG | This is the serial instruction line. Although there are 16 bits transferred during S0 – S15 only the bits during S3-S15 are valid (S3=LSB). The first 2-bits are “DONT-CAREs” The definitions of the instructioncodes are described later on, in a couple of tables. IRG is a unidirectional (tristate) line that switches between Vss and Vdd |  | Vss-Vdd (3ST) |
|  | EXT | This is the serial data line. It functions similar to to the IRG-line but valid bits are only transfered during S3 – S9 (S3=LSB) It also is unidirectional and also switches between Vss and Vdd. The EXT-line will transfer several types of data between the chips and character data to the Printer. |  | Vss- Vdd (3ST) |
|  | I/O-1, I/O-2, I/O-4, I/O-8 Databus | These lines transfer BCD-digit data between the CPU and the RAM-chips. It could in pricipal transfer a few HEX-codes as well but there is not a lot of documentation on this. During the S0-S15 state-times (1 instructioncycle) a full 16-digit value can be tranfered. Likewise 8 2-Digit user-programssteps can be transfered (See Value-representation Diagram) These lines switch between Vss and Vdd and are tristate |  | Vss- Vdd (3ST) |
| Scanningbusses | D0-D15 | The are the Scan-send-lines. During subsequent instruction cycles each of the D-lines will go high (Vss) and stay high during that instructioncycle until S14. However, the leading edge of the D-signal will start during S15 in the previous instructioncycle The D-Lines scan the keyboard and some will be buffered to drive the Display LED-digits. (L-Bus). The D-lines will switch between Vss and Vdd, they are output only. |  | Vss- Vdd (out) |
|  | L1 - L12 | Derived from D2 –D13 these lines are buffered by 2 SN27882-chips to enhance the power output of the scanlines in order to drive the Display LED-digits. They switch between Vss and –Vbat |  | Vss- -Vbat (out) |
|  | KN – KT | These are the Scanning-return-lines with the exeption of KR. They will connect to a D-line when a key is pressed on the keyboard and some extra K-lines are user as jumper or switch lines for special functions like the cardreader enable and position switch and one to distinguish between a TI59 and a TI58 (Cardreader present or not) On some models a 1200 ohm resistor is placed in series with these lines. Some connections between D & K lines may lead to undocumented TI59/58 features like: KQ-D15=Master Reset KN-D5 … D11= Fast Mode/Firmware |  | (IN) |
|  | BUSY (KR) | This Scan-returnline is in effect a BUSY line for the Printer and The Cardreader. The rest of the machine will wait as long as this line is held at Vss by a periferal device |  | (IN) |
|  | SA – SH & DPT | Segment-lines will drive the 7-Segment Display LEDs of each digit. SH only drives the first led digit to display the “[“ symbol when the CPU is in calculation mode. Some TI59/58’s and the TI58C have this line buffered by an extra transistor to enhance the output (remember that the D-lines are high only one quarter state-time on during calculation mode). Although my documentation was not clear on this I suspect the Segment-lines to be driven between Vss and –Vbat |  | ??? Vss- -Vbat (out) |
| Powerbus | Vss | From Battery | 0 Volt |  |
|  | -Vbat | From Battery | -3.7 Volt |  |
|  | Vdd | From Powermodule | -10 Volt |  |
|  | Vgg | From Powermodule | -15.7 Volt |  |

The following diagram shows a simplified representation of the way the calculator is wired (power, cardreader and printerconnections are not shown)

### Abreviated keyboardscanning and displaydrive circuit

Internal connect

(jumper/switch)

TI59/58 CPU CORE

Keyboard

Display

driver

D-BUS

K-BUS

S-BUS

1 & 2 Clock timingdiagram

Below a small version of the TI59/58 Powersupply diagram. (A larger version is aviable at my website)

The Scanning ROMs

The table below shows the RAM-registers in the scanning ROMs. Beside RAM the Scanning ROMs also contain 32 Constants in ROM and 2.5Kb “Second” ROM each.

“First ROM” is part of the firmware ROM (TMC 0571). Beside the obvious difference between Constant-ROM and the other types, the documentation I have tells little about the difference between First & Second ROM. One may conclude however, that the First-ROM -being equal to the Library-ROM in connection method- contains the preprogrammed functions (like statistical functions et. al.) and the Second ROM’s in the scanning ROMs may contain code for more primitive functions as a kind of microcode for the Arithmetic-chip.

TI59 CPU-registers

|  | ###### Digit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMC0582 Registers | 0 | 10 User Flags |  |  |  |  | 0 | 0 | 0 | RAM address Second ROM Address |  |  | Byte | Prog SRC FLG | Last Key |  | Fix ptd |  |
|  | 1 | Mantissa |  |  |  |  |  |  |  |  |  |  |  |  | Exponent |  | Sign | Hierarchy Stack |
|  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TMC0583 Registers | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 9 | List of dataflag | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Current Page |  | New Page |  | Security Code | No of ROMs | No of prog banks |  |
|  | A | RAM or CONSTANT ROM PROGRAMCODES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | B | T-REGISTER Mantissa |  |  |  |  |  |  |  |  |  |  |  |  | Exponent |  | Sign |  |
|  | C | PC 1 | OP 1 | PC 2 | OP 2 | PC 3 | OP 3 | PC 4 | OP 4 | PC 5 | OP 5 | PC 6 | OP 6 | PC 7 | OP 7 | PC 8 | OP 8 | Opcode & Par. Count for Hierarchy stack |
|  | D | Page in run | 0 | 0 | 0 | 0 | 0 | 0 | RAM memory min. address |  |  | RAM memory max. address |  |  | No of pending ops ops. | Parentheses count | Deg Rad Grd |  |
|  | E | Level 6 RAM adress Const &2nd ROM Address |  |  | Byte NR | Prog SRC FLG | Level 5 RAM adress Const &2nd ROM Address |  |  | Byte NR | Prog SRC FLG | Level 4 RAM adress Const &2nd ROM Address |  |  | Byte NR | Prog SRC FLG | Cond. Rtn Flag | Super Routine Stack |
|  | F | Level 3 RAM adress Const &2nd ROM Address |  |  | Byte NR | Prog SRC FLG | Level 2 RAM adress Const &2nd ROM Address |  |  | Byte NR | Prog SRC FLG | Level 1 RAM adress Const &2nd ROM Address |  |  | Byte NR | Prog SRC FLG | No of SBR levels |  |